CONFIGURATIONS

1 SW2- Access Switch.

Creating the VLANs department, assigns access ports, and configures its half of the EtherChannel bundle up to SW1.

enable

configure terminal

hostname SW2

Create VLANs

vlan 10

name Administration

vlan 20

name Production

vlan 30

name Warehouse

vlan 40

name Servers_Future

vlan 99

name Management

exit

Access ports

interface FastEthernet0/1

switchport mode access

switchport access vlan 10

spanning-tree portfast

no shutdown

exit

interface range FastEthernet0/2, FastEthernet0/6

switchport mode access

switchport access vlan 20

spanning-tree portfast

no shutdown

exit

interface FastEthernet0/3

switchport mode access

switchport access vlan 30

spanning-tree portfast

no shutdown

exit

EtherChannel (LACP active/active) to SW1

interface range GigabitEthernet0/1-2

channel-group 1 mode active

no shutdown

exit

interface Port-channel1

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40,99

exit

Management SVI (for remote switch management via VLAN 99)

interface vlan 99

ip address 172.30.18.226 255.255.255.224

no shutdown

exit

ip default-gateway 172.30.18.225

Harden unused ports

interface range FastEthernet0/4, FastEthernet0/5, FastEthernet0/7-24

shutdown

exit

end

2 SW 1- Central Hub

enable

configure terminal

hostname SW1

vlan 10

name Administration

vlan 20

name Production

vlan 30

name Warehouse

vlan 40

name Servers_Future

vlan 99

name Management

exit

EtherChannel (LACP active/active) to SW2

interface range GigabitEthernet0/1-2

channel-group 1 mode active

no shutdown

exit

interface Port-channel1

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40,99

exit

Trunk to R1 (adjust interface number to match topology)

interface GigabitEthernet0/3

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40,99

no shutdown

exit

Management SVI

interface vlan 99

ip address 172.30.18.227 255.255.255.224

no shutdown

exit

ip default-gateway 172.30.18.225

end

R1- EDGE ROUTER

enable

configure terminal

hostname R1

Physical interface stays up, no IP itself

interface GigabitEthernet0/0

no shutdown

exit

Sub-interfaces (router-on-a-stick)

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 172.30.18.1 255.255.255.192

exit

interface GigabitEthernet0/0.20

encapsulation dot1Q 20

ip address 172.30.18.65 255.255.255.192

exit

interface GigabitEthernet0/0.30

encapsulation dot1Q 30

ip address 172.30.18.129 255.255.255.192

exit

interface GigabitEthernet0/0.40

encapsulation dot1Q 40

ip address 172.30.18.193 255.255.255.224

exit

interface GigabitEthernet0/0.99

encapsulation dot1Q 99

ip address 172.30.18.225 255.255.255.224

exit

WAN interface to ISP

interface GigabitEthernet0/1

ip address 172.30.19.1 255.255.255.252

no shutdown

exit

Default route out to ISP

ip route 0.0.0.0 0.0.0.0 172.30.19.2

end

configure terminal

Enable SSH version 2 only

ip ssh version 2

ip ssh time-out 60

ip ssh authentication-retries 3

VTY lines: SSH only, no Telnet

line vty 0 4

transport input ssh

login local

exec-timeout 5 0

exit

restrict SSH source to management VLAN via ACL

access-list 10 permit 172.30.18.224 0.0.0.31

line vty 0 4

access-class 10 in

exit

end

Checking for results

On SW1 / SW2

show etherchannel summary - Po1 should show (P) on both Gi0/1 and Gi0/2, protocol LACP

show vlan brief  -confirms access port-to-VLAN mapping

On R1:

show ip interface brief -all sub-interfaces up/up with correct IPs

show ip route - connected routes for all 5 subnets + default route

show ip ssh - confirms SSH v2 enabled

SW1

SW2

R1